| DE-001 |  |  |  | ge | The nature of UB should clarified to be consistent with its use in the standard (e.g. Annex L) and to clarify the requirements for observable behavior before UB. | Adopt the clarification (one note to the definition of UB) from WG 14 N3128. | *Defer to get input from Martin |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DE-002 |  |  |  | ge | The clarification of pointer provenance should be integrated into C23. | Integrated the model in the latest version WG N3005 + editorial corrections. | *Defer to get input from Martin see also FR-238 |  |
| US-003 |  | 2 | 10 | ed | ###### The Unicode Consortium has provided a correction to this reference. | Change “Unicode Character Database” in the title to “Unicode Identifier and Pattern Syntax”. Change the editors to Mark Davis and Robin Leroy. | Accepted |  |
| US-004 |  | 2 | 9 | ed | ###### The Unicode Consortium has provided a correction to this reference. | Delete “and Laurentiu Iancu” because the latest edition has one less author. | Accepted |  |
| GB-005 |  | 3 |  | te | Since ISO 2382 defines “negate” to be a logical, not arithmetic operation, we need a definition of arithmetic negation, which then needs to be used in appropriate places. | After 3.3, add a new subclause defining “arithmetically negate” as “return the negative of a number”, with a Note: “For a floating-point number, this logically negates the sign bit; for an integer, this is equivalent to subtracting from zero.”. Change “negated” to “arithmetically negated” and “negates” to “arithmetically negates” in the following places: 6.4.4.1 paragraph 8 (three times), 6.4.4.2 paragraph 12 (twice with “negated” and once with “negates”), 7.24.1.5 paragraph 4, 7.24.1.6 paragraph 4, 7.31.4.1.2 paragraph 4, 7.31.4.1.3 paragraph 4. Note: do not change the Table in F.3, because “negate” there is the name of an operation from IEC 60559, so needs to remain unchanged. | Accepted with comment: Change “return” to “produce” and use the note from N 3135 in place of the note in this comment. |  |
| US-006 |  | 4 | 6 | te | ###### The inclusion of <string.h> in freestanding has exclusions for functions that are difficult to support in freestanding. When investigating supporting this in practice, we were asked to exclude strtok from the list of required interfaces to support. The requirement for it to support “global state” makes it a burden to implement efficiently in all circumstances. | Add strtok to the list of exceptions in p6. | Accepted |  |
| US-007 |  | 4 | Para 7 | ed | ###### “ON” in the last sentence should be lower case and not bold, as in all other occurrences in the standard (e.g. 7.3.4 #2). | Change “ON” to “on”. | Accepted |  |
| US-008 |  | 5.1.2.3 | 12 | ed | ###### “single-precision” was not replaced with “float”, as prescribed in CD NB comment GB-007. | Change “single-precision” to “float”. | Accepted with comment: Resolved by GB-009. |  |
| GB-009 |  | 5.1.2.3, 5.2.4.2.2 |  | ed | The terms “single-precision” and “double-precision” are not defined in this document and the definitions in ISO/IEC 2382 (in terms of use of one or two computer words to represent a value) are not appropriate for their uses in this document because there is no guarantee that the types float or double correspond to those definitions. Furthermore, IEC 60559 refers to the formats as binary32 and binary64, not single and double. This was comment GB-007 on CD1, which was accepted as modified in N3082; N3082 said “Agree with first two changes.”, but those first two changes were not applied, only the changes to 6.7.1 from N3082. | In 5.1.2.3 paragraph 12, change “single-precision” to “float” and “double-precision” to “double” (in both cases formatted as C keywords). In 5.2.4.2.2 paragraph 29, change “single-precision” to “binary32” and “double-precision” to “binary64”. Also ensure associated index entries for the “single-precision” and “double-precision” terms are removed. | Accepted |  |
| GB-010 |  | 5.2.1 | 3 | te | CD1 comment US-011 was accepted with a fix to wording alternative 1 from N3046, but the accepted wording has not been applied. | At end of paragraph, add “The $ character is reserved for use in identifiers as an implementation extension.”. | *Defer for input from Corentin (see also US-032) |  |
| GB-011 |  | 5.2.1, 5.2.1.1, M.2 |  | ed | In 5.2.1 footnote 17, and 5.2.1.1, and the entry in M.2 for new characters, the quotes around “Backtick” should be left and right quotes, not vertical quotes. | Change to left and right quotes (LaTeX `` and ‘’), in all three places. | Accepted |  |
| GB-012 |  | 5.2.1, 5.2.1.1, M.2 |  | ed | In 5.2.1 footnote 17, and 5.2.1.1, and the entry in M.2 for new characters, the backtick is illustrated with a mark slanting to the right, when it should slant to the left (maybe that mark is a fixed-width version of an opening quote, but it should actually be a spacing grave accent, not an opening quote). | Change the backtick to an actual spacing grave accent, in all three places. | Accepted |  |
| US-013 |  | 5.2.4.1 | 1 | ed | NB comment GB-012 from [n3019] identifies the issue that the 2011 edition of ISO/IEC 10646 removed eight-digit short identifiers that were present in the 2003 edition (and this removal still ###### applies as of the 2020 edition) but the CD2 draft supports eight-digit short identifiers but not six-digit short identifiers. See N 3111 for more information. | change “0000FFFF” to “00FFFF” and change “00010000” to “010000”. — 31 significant initial characters in an external identifier (each universal character name specifying a short identifier of 0000FFFF or less is considered 6 characters, each universal character name specifying a short identifier of 00010000 or more is considered 10 characters, and each extended source character is considered the same number of characters as the corresponding universal character name, if any)18) | Accepted |  |
| GB-014 |  | 5.2.4.1, 6.4.3 |  | ed | The 2011 edition of ISO/IEC 10646 removed eight-digit short identifiers that were present in the 2003 edition (and this removal still applies as of the 2020 edition). Thus, the C standard should only use short identifiers with no more than six digits. This was CD1 comment GB-012, which was rejected because WG14 didn’t like the proposed wording rather than because of disagreement with the existence of the editorial issue (use of the term “short identifier” inconsistently with the normative reference defining that term). | Apply the changes shown in N3111. | Accepted |  |
| US-015 |  | 5.2.4.2.2 | 15 | ed | There is a space or bad line break in front of the comma “,” after the first “DEC_EVAL_METHOD”. | Remove the space or bad line break in front of the comma “,” after the first “DEC_EVAL_METHOD”. | Accepted |  |
| US-016 |  | 5.2.4.2.2 | 18 | ed | There is a space or bad line break in front of the comma “,” after “FLT_IS_IEC_60559”. | Remove the space or bad line break in front of the comma “,” after “FLT_IS_IEC_60559”. | Accepted |  |
| GB-017 |  | 5.2.4.2.2 | 21 | ed | As was noted when the N3082 changes for CD1 comments US-075 and GB-279 were discussed in WG14, “must be” should be changed to “is”. | Change “must be evaluated” to “is evaluated”. | Accepted |  |
| GB-018 |  | 5.2.4.2.2 | 21 | ed | A space is missing after “.”. | Change “NaN.If” to “NaN. If”. | Accepted |  |
| US-019 |  | 5.2.4.2.2 | 21 | ed | A space is missing before “If an optional …”. | Insert a space before “If and optional …”. | Accepted |  |
| US-020 |  | 5.2.4.2.2 | 29 | ed | “single-precision” and “double-precision” were not changed to “binary32” and “binary64”, as prescribed in CD NB comment GB-007. | Change “single-precision” and “double-precision” to “binary32” and “binary64”, respectively. | Accepted with comment: Resolved by GB-009. |  |
| US-021 |  | 5.2.4.2.2 | 29 | ed | The given decimal floating constants are not correct if the evaluation method evaluates float and double constants with extra range or precision. Even with FLT_EVAL_METHOD = 0, they might not yield the correct value when the macro is used within the scope of a FENV_ROUND pragma. The given hexadecimal floating constants do not have these problems. | Change: “The following describes floating-point representations that also meet the requirements for binary32 and binary64 numbers in IEC 60559,29) and the appropriate values in a <float.h> header for types float and double:” to: “The following describes floating-point representations that also meet the requirements for binary32 and binary64 numbers in IEC 60559,29) and the appropriate values in a <float.h> header for types float and double. Note that the decimal floating constants might not give correct values (and hence are not appropriate values in a <float.h> header) if FLT_EVAL_METHOD is not 0 or if a translation-time rounding mode other than the IEC 60559 default is supported (either as the default or as a constant rounding mode set by an FENV_ROUND pragma). The hexadecimal floating constants are correct in all such cases because their values are exactly representable in the type.” | Accepted |  |
| CA-022 |  | 5.2.4.2.2 | Paragraph 18 | te | The definition fails to account for the possibility of types with the same p, emin, and emax but a different base (b). Also, the committee’s intent regarding values other than normalized floating point numbers should be reinforced with a note. | Edit as follows: Whether a type has the same <ins>base (b), </ins>precision (p)<ins>,</ins> and exponent range [ … ] Add a note following the content of paragraph 18: NOTE Outside of the normalized floating point numbers, the representability of values (e.g., negative zero) of the IEC 60559 format is not implied. | Accepted with comment: Change “floating point” to “floating-point”. |  |
| GB-023 |  | 5.2.4.2.3 | 8 | ed | The “min” and “max” subscripts in the table should not be in italics. | Change those subscripts out of italics (use LaTeX \min and \max). | Accepted |  |
| US-024 |  | 5.2.4.2.3 | Preferred quantum exponents table | ed | The Operation column contains C operations, not mathematical ones. Thus, hyphen in program font should be used instead of minus math symbol, and likewise “+” in program font should be used instead of plus math symbol. | In the Operation column, in the row “−(x), +(x)”, use hyphen in program font instead of minus math symbol, and use “+” in program font instead of plus math symbol. | Accepted |  |
| DE-025 |  | 6.2 | 8 | ed | The new text for compound literals does not use the terms scope consistently with its definition. Scopes are defined for identifiers or type names but do not exist independently. | 6.2p8 A compound literal (which is an expression that provides access to an anonymous object) is associated with the enclosing scope of the type name used in its definition that corresponds to the placement of the compound literal in the program; that scope is either file scope, function prototype scope, or block scope. | Accepted |  |
| GB-026 |  | 6.2.4 | 2 | ed | There is a missing “.” at the end of the sentence to which footnote 37 is attached. (This was part of comment GB-025 on CD1, which was accepted, but apparently that part of the fix was not applied.) | Add “.” after “throughout its lifetime”. | Accepted |  |
| DE-027 |  | 6.2.7 | 3 | te | A composite type for structure, unions, and enumerated types should be defined for use in the conditional operator (and elsewhere). | It is suggested to adopt the wording from WG14 N3122 (changes 1, 2, 3a or 3b). | *Defer for input from Martin see also GB-028, DE-029 |  |
| GB-028 |  | 6.2.7 | 3 | te | Composite types are not specified for compatible structure or union or enumerated types; this issue was raised in CD1 comment DE-031, but the wording there was not ready for inclusion. | Apply the changes given in N3122. | *Defer for input from Martin |  |
| DE-029 |  | 6.3.1.8 | 1 | te | The conversions of enumerated type should be clarified. | It is suggested to adopt the wording from WG14 N3122 (change 4). | *Defer for input from Martin |  |
| FR-030 |  | 6.3.1.8 | p1 | te | ###### With the different changes to enumeration types, there is now more confusion how these behave when used in arithmetic. Make the conversion to the underlying type explicit. | Add the text to “usual arithmetic conversions” as follows (similar to n3122): Otherwise, <ins>if any of the two types is an enumeration it is converted to its underlying type. Then,</ins> the integer promotions are performed on both operands. Then the following rules are applied to the promoted operands: | *Defer for input from Martin |  |
| GB-031 |  | 6.3.2.4 | 1 | ed | The word “The” should not start with an uppercase letter in the middle of a sentence. | Change “The result” to “the result”. | Accepted |  |
| US-032 |  | 6.4 | 3 | te | "N2701: @ and $ in source and execution character set" has some undesirable effects on the specification of universal character names. | Proposed changes are in N3124 | *Defer until Thursday for Corentin (see also GB-010) |  |
| US-033 |  | 6.4.3 | 4 | ed | NB comment GB-012 from [N3019] identifies the issue that the 2011 edition of ISO/IEC 10646 removed eight-digit short identifiers that were present in the 2003 edition (and this removal still applies as of the 2020 edition) but the CD2 draft supports eight-digit short identifiers but not six-digit short identifiers. See N 3111 for more information. | Replace: The universal character name \Unnnnnnnn designates the character whose eight-digit short identifier (as specified by ISO/IEC 10646) is nnnnnnnn. Similarly, the universal character name \unnnn designates the character whose four-digit short identifier is nnnn (and whose eight-digit short identifier is 0000nnnn). With: A universal character name designates the character in ISO/IEC 10646 whose code point is the hexadecimal value represented by the sequence of hexadecimal digits in the universal character name. [Editor’s note: Remove footnote 80] | Accepted |  |
| US-034 |  | 6.4.4.2 | Para 8 | ed | The last sentence of 6.4.4.2 #8 says, “All floating constants of the same source form83) shall convert to the same internal format with the same value.” However, with the FENV_ROUND and FENV_DEC_ROUND pragmas this is no longer true. 7.6.2 #4 for FENV_ROUND says, "Floating constants (6.4.4.2) of a standard floating type that occur in the scope of a constant rounding mode shall be interpreted according to that mode.” The proposed change is editorial in the sense that it is an implication of the new pragmas. | Change the last sentence of 6.4.4.2 #8 to, “All floating constants of the same source form83) shall convert to the same internal format and, provided they are subject to the same translation-time rounding direction (either the default or a constant rounding mode set by an FENV_ROUND or FENV_DEC_ROUND pragma), to the same value.” | Accepted |  |
| GB-035 |  | 6.4.4.4 | 15 | ed | The wording “A wchar_t character constant prefixed by the letter L” is redundant and inconsistent with the previous paragraphs which don’t mention the prefix for each kind of character constant. | Change “A wchar_t character constant prefixed by the letter L” to “A wchar_t character constant”. | Accepted |  |
| US-036 |  | 6.5 | 7 | te | The aliasing rules Subclause 6.5, paragraph 7 allows “the signed or unsigned type” to alias another object but fails to mention type compatibility. Consequently, it is unclear if a compatible enumeration type can alias such an object. See N 3112 for more information. | Change: — a type that is the signed or unsigned type corresponding to the effective type of the object, — a type that is the signed or unsigned type corresponding to a qualified version of the effective type of the object, to: — the signed or unsigned type compatible with the underlying type of the effective type of the object, — the signed or unsigned type compatible with a qualified version of the underlying type of the effective type of the object, | Accepted |  |
| GB-037 |  | 6.5.15 |  | te | CD1 comment DE-117 pointed out an issue with semantics for conditional expressions where one argument has variably modified type and the other is a null pointer constant or has type nullptr_t, which can result in the type of the conditional expression depending on a VLA size expression that is not evaluated. That case should be explicitly undefined behavior, similar to the case of a composite type involving a size expression that is not evaluated; unlike that other changes that were proposed in that (rejected) CD1 comment, addressing this does not involve other tricky semantic issues. | After paragraph 7, insert a new paragraph: “If one operand is a pointer to a variably modified type and the other operand is a null pointer constant or has type nullptr_t, the behavior is undefined if the type depends on an array size expression that is not evaluated.”. | Accepted |  |
| GB-038 |  | 6.5.16.1 | footnote 127 | ed | This footnote is no longer correct and should be removed. See the January minutes under “US 9-034 and other nullptr comments”, where “removing footnote 126” (as it was then) was approved (despite that approval, it seems the footnote removal didn’t get into the disposition of comments document). | Remove this footnote. | Accepted |  |
| GB-039 |  | 6.5.2.5 | 4 | ed | A cross-reference is shown as “??”. | Change “??” to “6.2.1” (as an appropriate internal link). | Accepted |  |
| GB-040 |  | 6.5.2.5 | 5 | ed | Some accepted changes from N3090 were not applied to this paragraph. | After “provides”, insert “access to”. Before “and other properties”, insert “initializer,”. | Accepted |  |
| GB-041 |  | 6.5.2.5 | 5 | ed | The first sentence could be clearer that the “function prototype scope” part qualifies everything that follows. In the discussion of N3090 at the January meeting, Aaron Ballman requested this be changed editorially to use bullet points for that reason, although that didn’t make the draft minutes. | Change the first sentence to use bullet points: “For a compound literal associated with function prototype scope: * the type is determined as if in block scope and no object is created; * if it is a compound literal constant it is evaluated at translation time; * if it is not a compound literal constant, neither the compound literal as a whole nor any of the initializers are evaluated.” | Accepted |  |
| GB-042 |  | 6.5.2.5 | 5 | ed | A space is missing between “a” and “compound literal”. | Change “acompound literal” to “a compound literal”. | Accepted |  |
| GB-043 |  | 6.5.2.5, 6.7, 6.7.1, 6.7.6.3 |  | ed | “storage class specifier” should be “storage-class specifier” (with a hyphen) for consistency. | Change “storage class specifier” to “storage-class specifier” in the following places: 6.5.2.5 paragraph 4, 6.7 Forward references, 6.7.1 paragraph 4, 6.7.6.3 paragraph 12. | Accepted |  |
| GB-044 |  | 6.5.9 | 4 | ed | There is no “.” at the end of the sentence to which footnote 121 is attached. (This was CD1 comment GB-072, which was accepted, but apparently that fix was not applied.) | Add a “.” before the footnote number. | Accepted |  |
| US-045 |  | 6.6 | 14 | te | The changes introduced by N2713 do not match existing implementation practice in many areas and is a breaking change for some implementations. Please see N3125 for details. | Proposed changes are in N3125 | *Defer for homework from Aaron Ballman. |  |
| DE-046 |  | 6.7 |  | te | VLA types can not be used anymore for arguments in some specific cases, which is a problem for code transitioning to C23 and also prevents size annotations to be added to legacy APIs. | Forward parameter declarations should be added as described in WG14 N3121. | *Defer for homework from Martin see also FR-073 |  |
| DE-047 |  | 6.7 | 2 | te | The meaning of the phrase a “declaration declares identifiers” is not clear (cf. WG14 N3070). | See WG14 N3123 for suggested changes (the type name special case may sbe needed for compound literals). | see also GB-048, DE-049, GB-050, GB-069*, GB-072 |  |
| GB-048 |  | 6.7 | 2 | te | As discussed in N3070, this paragraph leaves it unclear exactly what is meant by a declaration declaring a declarator, tag of members of an enumeration. | The direction previously given by WG14 was to disallow questionable cases. Replace this paragraph by the following to do so: “If a declaration other than a static_assert or attribute declaration does not include an init declarator list, its declaration specifiers shall include one of the following: * a struct or union specifier or enum specifier that includes a tag, with the declaration being of a form specified in 6.7.2.3 to declare that tag; * an enum specifier that includes an enumerator list.” Also add the following Example: The following are invalid, because the declared tag or enumeration constants are in a nested construct, rather than a declaration specifier of the declaration being of one of the given forms. struct { struct s2 { int x2a; } x2b; }; typeof (struct s3 { int x3; }); alignas (struct s4 { int x4; }) int; typeof (struct s5 *); typeof (enum { E6 }); struct { void (*p)(struct s7 *); }; [Example ends here.] (Note that N3123 gives alternative wording intended to address this issue. It does not seem clear that the N3123 version covers the case of a enumeration constant declared within a nested construct. Providing examples seems appropriate whatever change is made to normative text.) |  |  |
| DE-049 |  | 6.7 | 5 | te | The new constraint for auto breaks existing code that might transition from __auto_type to auto (which was intended to implement existing practice) and prevents useful coding patterns. The constraint was introduced to avoid problems for auto return types, but is not required for C23. Examples that should be allowed: // works with __auto_type auto x = (struct foo { int x; }){ }; // works as in C++ constexpr struct { double PI; double E; } constants = { .. }; // works with tag compatibility #define NEW(T) (T*)malloc(sizeof(T)) auto x = NEW(struct bar { int y; }); // works with ({ }) extension auto y = ({ goto end; end: 0; }); | The constraint should be removed completely or modified as suggested in n3123 (change 3). |  |  |
| GB-050 |  | 6.7 | 5 | te | As discussed in N3070, this paragraph leaves it unclear exactly what is meant by “declared identifiers”. | The direction previously given by WG14 (for the N3070 issues in general, except for case 4 where the requirement in question has been removed) was to disallow questionable cases. The following does that (N3123 provides alternative wording that allows more cases but has the problem that it leaves this paragraph not actually imposing any requirements because the definition of “declared” there wouldn’t allow any case where other kinds of identifiers are declared; the N3123 changes would also invalidate the Note in 6.7.9 paragraph 3 but do not include corresponding changes to that Note). Change “all declared identifiers” to “all identifiers declared by the declaration or anywhere within the sequence of tokens making up the declaration”. At the end of the Note in 6.7.9 paragraph 3, add: “The following is also invalid.” plus the example: auto alignas (struct s *) x = 0; At the end of 6.7.1, add “Because declarations using constexpr are underspecified, the following is invalid because tokens within the declaration declare s which is not an ordinary identifier.” plus the example: constexpr typeof (struct s *) x = 0; |  |  |
| GB-051 |  | 6.7.1 |  | ed | The accepted changes for CD1 comment GB-081 / N3071 were the entirety of the wording given under “About N3071” in N3082. Most of those were applied, but the lengthy Example given under “About N3071” in N3082 is missing. | Add that Example (shown as Example 4 in N3082). | Accepted |  |
| GB-052 |  | 6.7.1 | 17 | ed | In “initializer of small”, “small” should be in a fixed-width font. | Change “small” to a fixed-width font. | Accepted |  |
| GB-053 |  | 6.7.1 | 7 | ed | The example from CD1 comment GB-079 was accepted but doesn’t appear to have been added. | Add a new Example: constexpr int *p = {}; // Default initialization with a null pointer | Accepted |  |
| GB-054 |  | 6.7.1 | 7 | ed | Example 1 should not have been added in the given form; what was accepted for CD1 comment GB-080 was the N3078 wording, plus the text change suggested in GB-080 but without the example suggested in GB-080 because that example did not reflect the changes made in N3078 (specifically, the example from GB-080 assumes explicit initializers that are null but not null pointer comments are invalid, which is no longer the case). | Replace the example with that given as Example 1’ in N3078 under GB-080. (The actual code is the same in the two examples. However, the explanation for why the code is valid differs and the wording for the explanation in N3078 is now more appropriate.) | Accepted with comment: Duplicate of GB-052. |  |
| GB-055 |  | 6.7.1 | footnote 141 | ed | In “other constexpr declarations”, “constexpr” should be in a fixed-width font. | Change “constexpr” to a fixed-width font. | Accepted with comment: Duplicate of GB-051. |  |
| US-056 |  | 6.7.1 | Para 17 | ed | In the last sentence, “small” should be in program font. | In the last sentence, put “small” in program font. | Accepted |  |
| US-057 |  | 6.7.1 | Para 21 | ed | One of the changes to CD1 proposed in N3082 to address N3071 was to insert an example “After 6.7.1 #17 (EXAMPLE 3)” (referring to CD1). The proposed example is missing. | Insert the example proposed in N3082, after 6.7.1 #21 (EXAMPLE 5), as #22 EXAMPLE 6. | Accepted |  |
| GB-058 |  | 6.7.12.7 | 3 | ed | In “such a annotated function”, “annotated” should be removed (see CD1 comment GB-092, which seems to have been only partially applied). Failing that, “a” should be “an”. | Remove “annotated”. | Accepted |  |
| GB-059 |  | 6.7.12.7 | 4 | ed | “account as” seems unidiomatic: the last part of the changes for CD1 comment GB-093 has not been applied despite being accepted. | Change “account as store operations” to “are considered as store operations, for the purposes of these attributes”. | Accepted |  |
| GB-060 |  | 6.7.12.7.2 | 4 | ed | “property if” seems unidiomatic. (This was CD1 comment GB-096, apparently accepted but not applied.) | Change “property if” to “property of whether”. | Accepted |  |
| GB-061 |  | 6.7.12.7.2 | 8 | ed | “is in generally” is ungrammatical. | Change “is in generally” to “is in general”. | Accepted |  |
| GB-062 |  | 6.7.2.2 | 12 | ed | “a” should not appear before “bool”. | Change “a bool” to “bool”. | Accepted |  |
| GB-063 |  | 6.7.2.2 | 12 | ed | The changes from CD1 comment GB-099 do not seem to have been fully applied, although accepted: a comma should be “or” to make the text clearer. | Change “char, a” to “char or a”. | Accepted |  |
| GB-064 |  | 6.7.2.2 | 15 | ed | There is a missing space between sentences. | Change “arithmetic.Conversion” to “arithmetic. Conversion”. | Accepted |  |
| GB-065 |  | 6.7.2.2 | 7 | te | There appears to be no Constraint that would explicitly disallow declaring an enum with a fixed underlying type and then defining it without one. enum e : int; enum e { A }; (This was CD1 comment GB-105, which was accepted, but the changes don’t seem to have been applied.) | At the end of the Constraints in 6.7.2.2, add a new paragraph (after paragraph 7): “An enumeration with a fixed underlying type shall be defined with an enum type specifier. No enum specifier for an enumeration without a fixed underlying type shall include an enum type specifier.". |  |  |
| DE-066 |  | 6.7.2.3 | 8, Example 2 | ed | The last example in Example 2 redefining a enumeration constant is a valid example in a list of examples for invalid uses. | The last example redefining a enumeration constant should be removed or moved to Example 1 (duplicating the second-to last line which should remain in Example 2). | Accepted with comment: Move the last line to example 1 and copy the second-to-last line with it. |  |
| GB-067 |  | 6.7.2.5 | 5 | ed | Some words from CD1 comment GB-108 are missing here but would make the wording clearer. | Change “non-atomic unqualified type” to “non-atomic unqualified version of the type”. | Accepted |  |
| GB-068 |  | 6.7.2.5 | footnote 161 | ed | “type-name” should not be in a fixed-width font. This was CD1 comment GB-111, accepted but apparently not applied. | Change “type-name” in the footnote out of a fixed-width font. | Accepted |  |
| GB-069 |  | 6.7.4 | 3 | te | Similar to the cases discussed in N3070 (although this case isn’t discussed there), this paragraph leaves it unclear exactly what is covered by “contain” (in particular, when the tokens are outside the function body). | Twice in this paragraph, change “contain” to “contain, anywhere in the tokens making up the function definition”. After paragraph 11, add the following example: “The following inline definitions are invalid. static int a; typeof (a) inline f() { return 0; } typeof ((int) { 0 }) inline g() { return 0; } ” |  |  |
| US-070 |  | 6.7.6 | Para 7 | ed | There is extra space in “typedef s”. | Remove the extra space. | Accepted |  |
| GB-071 |  | 6.7.6.1 | 3 | ed | “variable” should not have been changed to “object” here. | Change “object” to “variable” (twice, in this paragraph, before the example code); “an object” should become “a variable”. Use of “object” after the example code is correct and should not be changed. | Accepted |  |
| GB-072 |  | 6.7.6.2 | 4 | te | As discussed in N3070, this paragraph leaves it unclear what cases are covered by the function prototype scope restriction on [*] declarators. The intended direction disallowing questionable cases was provided in C99 DR#341. | Apply the changes listed under “Change 4 (Case 3 in N3070)” in N3123, *but*, instead of the wording “as part of a parameter declaration”, say “as part of the nested sequence of declarators or abstract declarators for a parameter declaration, not including anything inside an array size expression in one of those declarators”. In addition, add a new Example after Example 4 in this subclause: The following is invalid, because the use of [*] is inside an array size expression rather than directly part of the nested sequence of abstract declarators for a parameter declaration: void f(int (*)[sizeof(int (*)[*])]); |  |  |
| FR-073 |  | 6.7.6.3 | p3 | te | ## The removal of function definitions with identifier list had the effect that some parameter declarations of VLA type cannot be expressed within C23 as currently proposed. The important use case here is that declarations of integer parameters come after the declaration of the VLA in question. We don’t need new syntax to handle this. | Prefix the paragraph with the following sentence: If an undeclared identifier ID is evaluated in the declaration of a function parameter P of a function definition, ID shall be declared later in the same parameter list as having integer type; the type of P is incomplete until the declaration of ID is completed. Alternatively, force the type to be size_t: If an undeclared identifier ID is evaluated in the declaration of a function parameter P of a function definition, ID shall be declared later in the same parameter list as having type size_t; the type of P is incomplete until the declaration of ID is completed. | *Defer for possible homework from Jens and/or Martin and discussion with DE-046. |  |
| GB-074 |  | 6.7.7 | 3 | ed | The text after item (h) should be part of that item, rather than being outside the itemized list. | Move “that has type unsigned int and an unspecified number of other parameters, returning an int.” up into item (h). | Accepted |  |
| GB-075 |  | 6.7.9 | footnote 178 | ed | “IS” should not be included in the reference to ISO/IEC 14882. | Change “ISO/IEC IS 14882” to “ISO/IEC 14882”. | Accepted |  |
| GB-076 |  | 6.7.9 | footnote 178 | ed | “commended” should be “recommended”. | Change “commended” to “recommended”. | Accepted |  |
| US-077 |  | 6.9 | 3 | te | A _Generic selection expression whose result is an integer constant is missing from the bulleted list. | Add a bullet before the final bullet: - part of the operand of a _Generic operator whose result is an integer constant; |  |  |
| US-078 |  | 6.9 | 5 | te | _Generic selection expressions are also missing from this paragraph. | Modify the paragraph by inserting _Generic in the serial list. |  |  |
| DE-079 |  | 6.9.1 | 10 | te | There is another missing case where evaluation of typeof should be required by the standard to conform to existing practice. See here for an example: https://godbolt.org/z/5hbdce3s3 | On entry to the function, the size expressions of each variably modified parameter and typeof operators used in declarations of parameters are evaluated and the value of each argument expression is converted to the type of the corresponding parameter as if by assignment. |  |  |
| GB-080 |  | 6.9.2 | 2 | ed | Before the bullet points, “with the composite type as of the end of the translation unit,” should be removed (that part of CD1 comment GB-127 was not properly applied). | Remove “with the composite type as of the end of the translation unit,” before “with an empty initializer”. | Accepted |  |
| GB-081 |  | 6.10.1 | 7 | ed | Part of the change for N3074 item 4 (CD1 comment FR-130) was not applied. | Change “The has_embed expression evaluates to” to “The has_embed expression evaluates to the same value as the following predefined macros (6.10.9.1)”. (Note that saying “predefined” instead of the originally suggested “mandatory” was accepted at the January meeting.) | Accepted |  |
| GB-082 |  | 6.10.1 | 8 | ed | “to be evaluate” should be “to be evaluated” (this part of CD1 comment GB-129 was not properly applied). | Change “to be evaluate” to “to be evaluated”. | Accepted |  |
| GB-083 |  | 6.10.3.1 | 18 | ed | “valid if i.dat produces 1 value,” should be present for both embeddings of i.dat, not just the first one (proper application of CD1 GB-134 was supposed to be adding it to the first one without removing from the second one). | Add “valid if i.dat produces 1 value,” to the comment on the second embedding of i.dat, without removing from the first one. | Accepted |  |
| GB-084 |  | 6.10.3.1 | 18 | ed | The exponent in the comment on the first #embed directive is badly formatted. | Change to copy the formatting (or rather lack thereof) in the comment on the second #embed directive. | Accepted |  |
| GB-085 |  | 6.10.4.5 | 8 | ed | In the example of valid redefinitions, the redefinition of FUNC_LIKE isn’t actually valid, because the first and second definitions lack white space separation present in the third one. This appears to be a previously unnoticed mistake introduced by the conversion to LaTeX for C17. This was submitted and accepted as CD1 comment GB-139. However, rather than that fix being applied in CD2, an additional second definition was added without any of the other changes listed being applied. | Remove the second definition that was added in CD2. Then, apply the changes listed for CD1 comment GB-139 (where “first”, “third” and “fourth” refer to positions after the spurious second definition has been removed, so first, fourth and fifth in CD2): Restore presence and absence of white space to the state it had in C11, for both the valid and the invalid definitions, as follows. Inside the replacement lists for the first, third and fourth FUNC_LIKE definitions, insert a space after ‘(‘ and one before ‘)’. Additionally, in the second FUNC_LIKE definition, insert a space after ‘(‘ and before ‘)’ in the listing of parameter names (not the replacement list), to illustrate, as in C11, that variation in the presence of space there is OK. | Accepted |  |
| GB-086 |  | 6.10.4.5 | 9 | ed | “and”, in the argument to showlist, should be in the same colour as the rest of that argument, since it’s not being used in any way as a standard macro. (An attempt was made in CD2 to fix the issue of it being in bold, which was reported in CD1 comment GB-140, but the colour still doesn’t match.) | Arrange for “and” to be shown in the same colour as the rest of that argument. | Rejected with comment: Not applicable to C23, but saved as a Gitlab issue for the next revision of the standard. |  |
| US-087 |  | 6.10.7 | 1 | ed | It is not clear what the word “ignore” means when discussing pragmas. In practice, compilers provide flags such as -Wunknown-pragmas which diagnoses unknown pragmas. This flag is used to catch defects resulting from misspelling the pragma name. | Replace: Any such pragma that is not recognized by the implementation is ignored. with: Each such pragma is a conditional feature that implementations need not support. Recommended practice Unrecognized pragmas should be diagnosed at translation time. | Accepted with comment: Wording from N 3135. |  |
| DE-088 |  | 6.11.5 | 1 | te | As discussed before, the intention is that auto becomes a type specifier. This should be made clear to implementors. | Add “Future standardization may change the auto storage-class specifier to a type specifier.“ |  |  |
| DE-089 |  | 6.11.X |  | te | The dangerous use of integers as null pointers should be discouraged. (otherwise, why have nullptr_t?) | Add a new item: “Null pointer constants of integer type are an obsolescent feature.“ Also add this exact text as a footnote to the first part of 6.3.2.3p3. |  |  |
| GB-090 |  | 7.1.2 | 1 | ed | The rule about types not including type qualifiers, in the final sentence of this paragraph, should apply to Annex H as well as to Annex K. (CD1 comment GB-147, which was accepted, would also have referred to Annexes F and G there, but those Annexes don’t actually define typedef names so there is a reasonable argument for not mentioning them in that sentence. But Annex H does define typedef names so definitely needs to be mentioned there.) | Change “here or in Annex K” to “here or in Annexes H or K” (using proper internal links for both Annex references). | Accepted |  |
| GB-091 |  | 7.1.2 | 1 | ed | The references to Annexes F, G, H and K in this paragraph, except for the last reference to Annex K, are not internal links as they should be (the Annex label is hardcoded in the LaTeX source instead of being a proper cross-reference). | Change those references into internal links. | Accepted |  |
| US-092 |  | 7.1.2 | 1 | ed | The last proposed change from CD NB comment GB-147 was not made. | Change “here or in Annex K” to “here or in Annexes F, G, H, or K”. | Accepted with comment: Resolved by GB-090. |  |
| US-093 |  | 7.1.2 | 1 | ed | There is a space or bad line break in front of the comma “,” after “__STD_IEC_559__”. | Remove the space or bad line break in front of the comma “,” after “__STD_IEC_559__”. | Accepted |  |
| DE-094 |  | 7.1.4 | 1 | te | There are two function-like macros that take a type name as one of the arguments (va_arg and offsetof). Type names may contain size expressions or typeof operators. In existing practice those are evaluated. | Add another item to the list: - Size expressions and typeof operators in type names which are passed as arguments to function-like macros are evaluated exactly once. |  |  |
| GB-095 |  | 7.2 | 3 | ed | “do not agree” should be “does not agree” (this editorial fix was noted in the discussion of CD1 comment CA-148 at the February meeting). | Change “do not agree” to “does not agree”. | Accepted |  |
| GB-096 |  | 7.2 | 3 | ed | “NDEBUG” should be in a fixed-width font. | Change “NDEBUG” into a fixed-width font. | Accepted |  |
| GB-097 |  | 7.2 | 4 | ed | Paragraph 4 is a duplicate of paragraph 2. | Remove paragraph 4. | Accepted |  |
| FR-098 |  | 7.2 | p1 | te | ## As formulated, the assert macro still leads to confusion to know whether or not under NDEBUG a call has to be syntactically correct, i.e. has exactly one argument after translation phase 4. | Replace the phrase (and adjacent code) If NDEBUG is defined as a macro name at the point in the source file where <assert.h> is included, the assert macro is defined simply as #define assert(...) ((void)0) by If NDEBUG is defined as a macro name at the point in the source file where <assert.h> is included, an invocation of the assert macro has no effect. |  |  |
| FR-099 |  | 7.2 | p3 | ed | ## This note could be clarified by indicating where the prototype in question could be found. | Remove the “1” from “Note 1”. Replace “by the given prototype” by “by the prototype as given below” | Accepted |  |
| FR-100 |  | 7.2 | p4 | ed | ## This paragraph is a duplicate of p2 | remove p4 | Accepted |  |
| GB-101 |  | 7.3.1 | 5, 6 | ed | “a arithmetic” should be “an arithmetic”. | Change “a arithmetic” to “an arithmetic” (in both paragraphs). | Accepted |  |
| GB-102 |  | 7.3.9.3 | footnote 247 | ed | “are equivalent expressions.” should not be in a fixed-width font. | Change “are equivalent expressions.”, at the end of this footnote, out of a fixed-width font. | Accepted |  |
| GB-103 |  | 7.3.9.6 | footnote 248 | ed | This footnote should be the same as footnote 247 (CD1 comment GB-151 requested a change be made to the footnotes for both creal and cimag, N3082 provided wording that was accepted, but the change has only been applied to cimag and not to creal). | Replace footnote 248 by a copy of footnote 247 (after the fix requested for that footnote in another comment has been applied). | Accepted with comment: Resolved by US-104. |  |
| US-104 |  | 7.3.9.6 | Para 2, Fn 248 | ed | The change in N3082 to address CD NB comment GB-151 was to be applied to two footnotes, in 7.3.9.2 and 7.2.9.6. It was applied in 7.3.9.2, but not 7.3.9.6. | Change footnote 248 to “For a complex variable z, z and CMPLX(creal(z), cimag(z)) are equivalent expressions. If imaginary types are supported, z and creal(z) + cimag(z)*I are equivalent expressions.” | Accepted |  |
| US-105 |  | 7.6.2 | Para 3-5 | ed | An FENV_ROUND pragma with direction FE_DYNAMIC doesn’t establish a constant rounding mode, but in a few places the specification implies or suggests it does. For example, 7.6.2 #3 has, “ … or the specified constant rounding mode is FE_DYNAMIC”. | In 7.6.2 #3, change : “direction shall be one of the names of the supported rounding direction macros for operations for standard floating types (7.6), or FE_DYNAMIC. If any other value is specified, the behavior is undefined. If no FENV_ROUND pragma is in effect, or the specified constant rounding mode is FE_DYNAMIC, rounding is according to the mode specified by the dynamic floating-point environment, …” to: “direction shall be: one of the names of the supported rounding direction macros for operations for standard floating types (7.6), to specify a constant rounding mode; or, FE_DYNAMIC, to specify dynamic rounding. If any other value is specified, the behavior is undefined. If no FENV_ROUND pragma is in effect, or the specified direction is FE_DYNAMIC, rounding is according to the mode specified by the dynamic floating-point environment, …” and, in the last sentence, change: “If the FE_DYNAMIC mode is specified and …” to: “If the direction FE_DYNAMIC is specified and …”. In 7.6.2 #4, change: “Within the scope of an FENV_ROUND pragma establishing a mode other than FE_DYNAMIC, …” to: “Within the scope of an FENV_ROUND pragma establishing a constant rounding, …” In 7.6.2 #5, delete “(other than FE_DYNAMIC)”. | Accepted with comment: Change “establishing a constant rounding” to “establishing a constant rounding mode”. |  |
| US-106 |  | 7.6.2 | Para 5 | ed | NOTE 1 in 7.6.2 says that constant rounding modes could be implemented using dynamic rounding modes as illustrated in the code segment in the note. However, this isn’t entirely true now, because dynamic rounding modes do not affect floating constants while constant rounding modes do. The proposed change is editorial in the sense that it is an implication of the new FENV_ROUND and FENV_DEC_ROUND pragmas. | In NOTE 1, in the first sentence, change “… as illustrated in the following example:” to “… as illustrated in the following example, except that this method does not interpret inexact floating constants according to the constant rounding mode as required.” | Accepted |  |
| FR-107 |  | 7.8 | p1 | ed | ## There now are also changes to <inttypes.h> so this also needs a VERSION macro. | Add the __STDC_VERSION_INTTYPES_H__ macro to the start section of <inttypes.h> and to the appropriate section of Annex B. | Accepted |  |
| GB-108 |  | 7.8.1 | 1 | te | For C23, we specified explicitly (N2562) for printf functions that formats such as %hhu that they behave as if using va_arg with a type determined by the default argument promotions and then convert to the type specified by the format (this defines what happens if the argument to %hhu isn't representable in unsigned char but is in the promoted type, for example). However, this relation remains unclear for the PRI* macros in <inttypes.h>: “suitable for use within the format argument of a formatted input/output function when converting the corresponding integer type” doesn't say anything about what type is used with va_arg or what conversions might occur, so doesn't make clear if there are defined semantics for using PRIu8 with a value outside the range of uint8_t, for example. | At the end of the paragraph, add the following wording: “The functions in the fprintf and fwprintf families shall behave as if they use va_arg with a type argument naming the type resulting from applying the default argument promotions to the type corresponding to the macro and then convert the result of the va_arg expansion to the type corresponding to the macro.”. |  |  |
| GB-109 |  | 7.8.1 | 3, 4, 5 | ed | The PRIb, PRIB and SCNb macros are shown in a different colour from the corresponding macros for other format conversion specifiers. | Change the formatting to be consistent. | Rejected with comment: Not applicable to C23, but saved as a Gitlab issue for the next revision of the standard. |  |
| GB-110 |  | 7.8.1 | 4 | ed | The cross-reference for %B for fwprintf is shown as “??” | Change “??” to “7.31.2.1” (as an appropriate internal link). | Accepted |  |
| GB-111 |  | 7.8.1 | 5 | ed | %b is an unsigned format, so the SCNb macros should be in the list of unsigned formats (paragraph 6 instead of paragraph 5). (The editing instructions in N3072 said to add them to paragraph 5, but that meant the paragraph 5 in CD1, which is now paragraph 6 after a new paragraph 4 was added with the PRIB macros.) | Move the SCNb macros from paragraph 5 to paragraph 6. | Accepted |  |
| GB-112 |  | 7.12 | 6, 7, 8, 9, 10 | ed | “a arithmetic” should be “an arithmetic”. | Change “a arithmetic” to “an arithmetic” (in all five paragraphs). | Accepted |  |
| US-113 |  | 7.12 | Para 3 | te | The definition of the *_t types refers to a type being at least as wide as another type. This terminology might suffice for numbers in the floating-point model (5.2.4.2.2), but does not imply inclusion for all the values that might be in a type (e.g. NaNs and infinities), as intended. The proposed specification (similar to that in 6.2.5 for the standard floating types) address this problem. | Change: “The types float_t double_t are floating types at least as wide as float and double, respectively, and such that double_t is at least as wide as float_t.” to: “The types float_t double_t are floating types such that the values of float and double are subsets of the values of float_t and double_t, respectively, and such that the values of float_t are a subset of the values of double_t.” | see also US-115 (duplicate) |  |
| US-114 |  | 7.12 | Para 3 | te | The float_t and double_t are not required to be real floating types. Thus, they might not have features defined for real floating types, e.g. they might not be usable as arguments for classification and comparison macros. The proposed solution, for the case where one of these types is not a real floating type, makes the behavior implementation-defend instead of implicitly unspecified. The alternative of requiring these types to be real floating types would seem to risk invalidating reasonable implementations. | To the paragraph, append the sentence: “If they are not real floating types, the behavior is implementation-defined.” | see also US-116 (duplicate) |  |
| US-115 |  | 7.12 | Para 3 | te | The definition of the *_t types refers to a type being at least as wide as another type. This terminology might suffice for numbers in the floating-point model (5.2.4.2.2), but does not imply inclusion for all the values that might be in a type (e.g. NaNs and infinities), as intended. The proposed specification (similar to that in 6.2.5 for the standard floating types) address this problem. | Change: “The types float_t double_t are floating types at least as wide as float and double, respectively, and such that double_t is at least as wide as float_t.” to: “The types float_t double_t are floating types such that the values of float and double are subsets of the values of float_t and double_t, respectively, and such that the values of float_t are a subset of the values of double_t.” |  |  |
| US-116 |  | 7.12 | Para 3 | te | The float_t and double_t are not required to be real floating types. Thus, they might not have features defined for real floating types, e.g. they might not be usable as arguments for classification and comparison macros. The proposed solution, for the case where one of these types is not a real floating type, makes the behavior implementation-defend instead of implicitly unspecified. The alternative of requiring these types to be real floating types would seem to risk invalidating reasonable implementations. | To the paragraph, append the sentence: “If they are not real floating types, the behavior is implementation-defined.” |  |  |
| US-117 |  | 7.12.1 | Para 5 | ed | Applying the change to address CD NB comment GB-164 prescribed in N3082 resulted in “If a floating result overflows, default rounding is in effect and the integer expression math_errhandling & MATH_ERRNO is nonzero, the integer expression errno acquires the value ERANGE.” The punctuation makes reading awkard. | Insert a comma “, “ after “default rounding is in effect”. | Accepted with comment: Wording from N 3135. |  |
| GB-118 |  | 7.12.6.2 | 2 | ed | The change from CD1 comment GB-173, to make byte order for various functions follow __STDC_ENDIAN_NATIVE__, was not applied here. | Change “The order of bytes in the array is implementation-defined.” to “The order of bytes in the array follows the endianness specified with __STDC_ENDIAN_NATIVE__ (7.18.2).”. | Accepted with comment: Apply to 7.12.16.2 instead of 7.12.6.2. |  |
| GB-119 |  | 7.12.7.2, 7.12.7.6, 7.12.7.8, 7.12.9.10, 7.12.9.11 |  | ed | The function synopses for the compoundn, pown, rootn, fromfp, ufromfp, fromfpx, ufromfpx functions include <stdint.h>, because those functions used to have prototypes using intmax_t or uintmax_t. Those types are no longer used in the prototypes, so the include should be removed from the synopses. | Remove the include of <stdint.h> from the listed function synopses. | Accepted |  |
| US-120 |  | 7.12.10.03 | Para 2 | te | The second sentence in the description of the remquo functions is, “In the object pointed to by quo they store a value whose sign is the sign of x/y and whose magnitude is congruent modulo 2^n to the magnitude of the integral quotient of x/y, where n is an implementation-defined integer greater than or equal to 3.” This requirement for the sign (and the absence of -0 in integer types) means that 0 cannot be stored when the integral quotient is equivalent mod 2^n to 0 but the sign of x/y is negative. This restriction is not needed and is contrary to common practice. | Change the sentence to “In the object pointed to by quo they store a value whose magnitude is congruent modulo 2^n to the magnitude of the integral quotient of x/y, where n is an implementation-defined integer greater than or equal to 3. If the value stored is not zero, its sign is the sign of x/y.” |  |  |
| US-121 |  | 7.12.14 7.12.14.01, 7.12.14.2, 7.12.14.3, 7.12.14.4, 7.12.14.5, 7.12.14.6 |  | ed | The specification of functions that round to narrower type (7.12.14) uses inconsistent terminology and its use of “format” might be confusing since non-Annex F implementations are allowed to return function results with extra range and precision. The proposed change is intended to use consistent terminology in the subclauses, and to avoid the use of “format”. | In 7.12.14 #1, change: “The functions in this subclause round their results to a type typically narrower301) than the parameter types.” to: “The functions in this subclause round their results to the return type, which is typically narrower301) than the parameter types.” In 7.12.14.1 #2, change: “These functions compute the sum of x + y, rounded to the type of the function. They compute the sum (as if) to infinite precision and round once to the result format, …” to: “These functions compute the sum of x + y, rounded to the type of the function. They compute the sum (as if) to infinite precision and round once to the return type, …” In 7.12.14.2 #2, change: “These functions compute the difference of x − y, rounded to the type of the function. They compute the difference (as if) to infinite precision and round once to the result format, …” to: “These functions compute the difference of x − y, rounded to the type of the function. They compute the difference (as if) to infinite precision and round once to the return type, …” In 7.12.14.3 #2, change: “These functions compute the product x×y, rounded to the return type of the function. They compute the product (as if) to infinite precision and round once to the result format, …” to: “These functions compute the product x×y, rounded to the type of the function. They compute the product (as if) to infinite precision and round once to the return type, …” In 7.12.14.3 #3, change: “These functions return the product of x × y, rounded to the return type of the function.” to: “These functions return the product of x × y, rounded to the type of the function.” In 7.12.14.4 #2, change: “These functions compute the quotient x ÷ y, rounded to the return type of the function. They compute the quotient (as if) to infinite precision and round once to the result format, …” to: “These functions compute the quotient x ÷ y, rounded to the type of the function. They compute the quotient (as if) to infinite precision and round once to the return type, …” In 7.12.14.5 #2, change: “These functions compute (x × y) + z (as if) to infinite precision and round once to the result format (the return type), …” to: “These functions compute (x × y) + z (as if) to infinite precision and round once to the return type, …” In 7.12.14.5 #3, change: “These functions return (x × y) + z, rounded to the return type of the function.” to: “These functions return (x × y) + z, rounded to the type of the function.” In 7.12.14.6 #2, change: “These functions compute the square root of x, rounded to the type of the function. They compute the square root (as if) to infinite precision and round once to the result format, …” to: “These functions compute the square root of x, rounded to the type of the function. They compute the square root (as if) to infinite precision and round once to the return type, …” In 7.12.14.6 #3, change: “These functions return the nonnegative square root of x, rounded to the return type of the function.” to: “These functions return the nonnegative square root of x, rounded to the type of the function.” | Accepted |  |
| US-122 |  | 7.12.16.02 | 2 | ed | One of the changes proposed in CD NB comment GB-173 was to be made in several sections. It was made in all the sections except 7.12.16.2. | Change “The order of bytes in the array is implementation-defined.” to “The order of bytes in the array follows the endianness specified with __STDC_ENDIAN_NATIVE__ (7.18.2).”. | Accepted with comment: Resolved by GB-118. |  |
| US-123 |  | 7.12.17 |  | ed | Pages 279-280: Many places have “(x)op (y)” so the formatting is strange. | Either add a space before “op” or remove space after “op”. | Accepted with comment: Resolved by GB-124. |  |
| GB-124 |  | 7.12.17.01, 7.12.17.2, 7.12.17.3, 7.12.17.4, 7.12.17.5 |  | ed | Various expressions have unusual formatting, with a space after a binary operator but not before that operator. | In all these subclauses, insert a space before all uses of the binary operators “<”, “<=”, “>”, “>=” and “¦¦” that are followed by a space. | Accepted |  |
| US-125 |  | 7.12.17.06 | Para 2 | te | 7.12.17, footnote 304 says, “Whether an argument represented in a format wider than its semantic type is converted to the semantic type is unspecified.” 7.12.17.6 #2 says, “The isunordered macro determines whether its arguments are unordered. It raises no floating-point exceptions if neither argument is a signaling NaN.” This disallows floating-point exceptions other than “invalid” from comparison with a quiet NaN. This seems to contradict the footnote, because conversion of arguments to semantic type might raise other floating-point exceptions, e.g. “overflow”. The other comparison macros don’t disallow exceptions other than “invalid” from comparison with a quiet NaN. The proposed change avoids this problem by using the same specification for exceptions for isunordered as for the other comparison macros. | In 7.12.17.6, change “It raises no floating-point exceptions if neither argument is a signaling NaN” to “It does not raise the "invalid" floating-point exception when x and y are unordered and neither is a signaling NaN”. |  |  |
| DE-126 |  | 7.16.1.1 | 2 | te | There is UB based on a specific syntactic construction of a pointer type. This is an anachronism which serves no useful purpose with typeof. | The parameter type shall be a type name specified such that the type of a pointer to an object that has the specified type can be obtained simply by postfixing a * to type.“ |  |  |
| US-127 |  | 7.16.1.1 | 2 | te | Type compatibility is also an issue with the va_arg macro. See N 3112 for more information. | Add text in green: — one type is compatible with a signed integer type, the other type is compatible with the corresponding unsigned integer type, and the value is representable in both types; |  |  |
| GB-128 |  | 7.16.1.4 | 4 | te | The rule that “Only the first argument passed to va_start is evaluated.” leaves unclear what might be done with arguments that do not expand to tokens, or that expand to unbalanced parentheses, which affects what implementation approaches might be valid for diagnostics following the recommended practice; the recommended practice refers to expansion in “unspecified contexts”, but that still needs to be consistent with the normative text. Undefined behavior in those limited cases seems appropriate to avoid constraining implementation approaches regarding whether and how they expand such preprocessing tokens. | At the end of this paragraph, add “If any additional arguments expand to include unbalanced parentheses, or a preprocessing token that does not convert to a token, the behavior is undefined.”. |  |  |
| GB-129 |  | 7.17.2 | 1 | ed | “Explicit, default initialization” is incorrect; explicit and default initialization are different things, and the sentence should apply to both of them, but the comma suggests it applies to a single kind of initialization that is both explicit and default. The change from “Explicit or default” to “Explicit, default” in commit 0fa7570f721e9367b10a6a3dbca26cfc40164000 “NB Comments 015-054” appears incorrect and not related to any of those comments. | Change “Explicit, default” to “Explicit or default”. | Accepted |  |
| GB-130 |  | 7.17.7.5 | 1 | ed | This paragraph needs to allow for the possibility that atomic_bool is different from _Atomic bool. (See CD1 comment GB-197; the particular form of the fix adopted for CD1 comment GB-195 means that an atomic_bool alternative needs to be included here.) | Insert “, atomic_bool,” before “or”. | Accepted |  |
| FR-131 |  | 7.18 |  | te | # The [[unsequenced]] attribute and the <stdbit.h> utilities have been introduced in parallel, and so their introduction has not been coordinated, yet. Since these bit functions are typical examples of pure functions (in the CS sense) we should provide all optimization hints available to us for their interfaces. | # Add the [[unsequenced]] attribute to all function and type-generic function interfaces in <stdbit.h> |  |  |
| FR-132 |  | 7.18.8 and 7.18.14 |  | ed | It seems that these two sets of interfaces describe exactly the same functions, namely resulting in 0 if x is 0 and otherwise 1 + ⌊ log₂ x ⌋ | Remove one of the two sections and the listing of the corresponding prototypes in Annex B. | *Defer for input from JeanHeyd |  |
| FR-133 |  | 7.18.9 and 7.18.6 |  | ed | It seems that these two sets of interfaces describe the same functions, and deviate only for the special case of 0. | Remove one of the two sections and the listing of the corresponding prototypes in Annex B. | *Defer for input from JeanHeyd |  |
| FR-134 |  | 7.18.10 and 7.18.05 |  | ed | It seems that these two sets of interfaces describe the same functions, and deviate only for the special case of 0. | Remove one of the two sections and the listing of the corresponding prototypes in Annex B. | *Defer for input from JeanHeyd |  |
| FR-135 |  | 7.18.16 | p2 and p3 | te | The bit ceiling family of functions is the only one in the new header <stdbit.h> that has undefined behavior if the mathematical result does not fit the target type. We found no rationale for that in the corresponding documents, so this probably has just been overlooked. The function has indeed a natural continuation in that situation, namely 0, which will not occur in other situations. | In p2 change … the behavior is undefined. to … return 0. In p3 append to the first sentence … or 0 if such a value is not representable in the return type. |  |  |
| GB-136 |  | 7.20.1 | 3 | ed | The quotes around “plain”, twice in this paragraph, should be proper left and right quotes, not straight quotes. | Change those quotes to left and right quotes (``plain’’ in LaTeX). | Accepted |  |
| DE-137 |  | 7.21 | 4 | te | It is undefined if the first argument of offset of defines a new type. But with typeof it is possible to wrap a type name with additional brackets, so this should be made more specific. offsetof(typeof(struct { int i, j; }), i); | If the specified type name contains a comma not between matching parentheses defines a new type or if the specified member is a bit-field, the behavior is undefined” |  |  |
| GB-138 |  | 7.22.5 | 1 | ed | A word from the resolution of CD1 comment GB-208 is missing. | Insert “converted” before “according”. | Accepted |  |
| GB-139 |  | 7.23.5.3 | 5 | ed | “teh” should be “the”. | Change “teh” to “the”. | Accepted |  |
| GB-140 |  | 7.23.5.3 | 6 | ed | “of increment” should be “of incrementing”. | Change “of increment” to “of incrementing”. | Accepted |  |
| GB-141 |  | 7.23.6.1 | 8 | te | AG Reference Bug 1647 (https://austingroupbugs.net/view.php?id=1647) ======= Currently printf("%lc", L'\0') is required to output no bytes. This is because 7.23.6.1 para 8 (c conversion) says "If an l length modifier is present, the wint_t argument is converted as if by an ls conversion specification with no precision and an argument that points to storage suitably sized for at least two wchar_t elements, the first element containing the wint_t argument to the lc conversion specification and the second a null wide character." This behavior is inconsistent with the related calls printf("%c", '\0'), wprintf("%lc", L'\0'), and wprintf("%c", '\0') which all output a NUL. It appears that most libc implementations have chosen to make all four consistent by having printf("%lc", L'\0') output a NUL; we are only aware of one (musl) that obeys a strict reading of the C standard. The Austin Group would like to require printf("%lc", L'\0') to output a NUL in the next POSIX revision (which will necessitate that it does not defer to C17 on this) but only if the C committee indicates that it will at least allow this behavior in C23. | Option 1 (require a NUL) - change the text to: If an l length modifier is present, the wint_t argument is converted as if by a call to the wcrtomb function with a pointer to storage of at least MB_CUR_MAX bytes, the wint_t argument converted to wchar_t, and an initial shift state. Option 2 (allow either behavior) - replace the period at the end of the current text with: , except that if the wint_t argument is a null wide character it is unspecified whether it is converted to no bytes or to a null byte. The Austin Group would prefer option 1. |  |  |
| GB-142 |  | 7.23.6.1, 7.31.2.1 |  | ed | The changes to recommended practice from N3072 item 15 to define ‘B’ as an optional normative feature were not fully applied. | In 7.23.6.1 paragraph 14, last sentence, replace all text after “implement it” by “as an option as described above.”. Likewise in 7.31.2.1 paragraph 14. | Accepted |  |
| GB-143 |  | 7.23.6.1, 7.31.2.1 |  | ed | It’s peculiar that the wording “The macro PRIBPTR from <inttypes.h> shall only be defined if the implementation follows the specification as given here.” refers to that specific macro. The statement would be true for any of the PRIB macros, but since uintptr_t is optional and missing uintptr_t support would be another reason for PRIBPTR not being defined, that’s not a particularly good one for a user to test to determine whether the %B feature is supported. | In 7.23.6.1 paragraph 8 and 7.31.2.1 paragraph 8, change “macro PRIBPTR” to “PRIB macros”. | Accepted |  |
| GB-144 |  | 7.23.6.1, 7.31.2.1 |  | te | The 0 flag should be ignored with precision by printf %b and %B, just as it is for other integer formats. | In 7.23.6.1 paragraph 6 and 7.31.2.1 paragraph 6, change “For d, i, o, u, x, and X conversions” to “For b, B, d, i, o, u, x, and X conversions”. |  |  |
| GB-145 |  | 7.23.6.1, 7.31.2.1 |  | ed | The formatting of references to conversion specifiers and their output is inconsistent. Some inconsistencies were fixed for CD1 comment GB-221, but CD2 also adds colour, meaning new inconsistencies are visible. | In 7.23.6.1 paragraph 6, ‘#’ flag, colour ‘b’ and ‘B’ the same as other conversion specifiers. In paragraph 7, each integer length modifier, colour ‘n’ the same as other conversion specifiers. In paragraph 13, use same colour for ‘e’ as for other conversion specifiers. Likewise in the same paragraphs in 7.31.2.1 (fwprintf). | Rejected with comment: Not applicable to C23, but saved as a Gitlab issue for the next revision of the standard. |  |
| GB-146 |  | 7.23.6.1, 7.31.2.1 |  | ed | Now that printf %B is normatively defined rather than just recommended practice, all the lists of formats that include %b should explicitly include %B as well. | Change “b” to “b, B” in lists of integer formats in the following places, in both 7.23.6.1 and 7.31.2.1: paragraph 4 (formats for precision), paragraph 6 (0 flag), paragraph 7 (hh, h, l, ll, j, z, t, wN, wfN length modifiers). | Accepted |  |
| GB-147 |  | 7.23.6.2, 7.31.2.2 |  | ed | “No null wide character is added.” (for %lc) is now a separate paragraph, presumably as a result of changes applied for CD1 comment GB-222. But it should be part of the previous paragraph, since it only makes sense in the context of “If an l length modifier is present”; it’s not something applying to %c in general, only the specific %lc sub-case, so should be in the paragraph for that sub-case. | Move that sentence back into the previous paragraph, while avoiding the previous problem of no space between the ‘.’ ending the previous sentence and the start of this sentence. | Accepted |  |
| GB-148 |  | 7.23.7.2, 7.31.3.2 |  | te | The description of fgets is unclear about the semantics when n is zero or negative, when “reads at most one less than the number of characters specified by n” would mean reading a negative number of characters. The same issue applies to fgetws. This was CD1 comment GB-224, which was accepted, but the accepted change doesn’t seem to have been applied. | Add “If n is negative or zero, the behavior is undefined.” to the end of 7.23.7.2 paragraph 2 and 7.31.3.2 paragraph 2 (this was the agreed resolution for the CD1 comment). Also add this to the list of undefined behavior in Annex J (the need for that was noted in the discussion of the CD1 comment). |  |  |
| GB-149 |  | 7.24.1.5 | 11 | ed | This paragraph has three paragraphs, each of which should have its own number. (This is part of CD1 comment GB-231, accepted but not fully fixed.) | Give all three paragraphs their own numbers. | Accepted |  |
| US-150 |  | 7.24.1.6 | Para 3 | ed | The second bullet ends in a period “.”, unlike the other bullets. | Remove the period “.” from the end of the second bullet. | Accepted |  |
| US-151 |  | 7.24.1.6 | Para 4 | ed | The second “s” in the last sentence should be italicized. | Italicize the second “s” in the last sentence. | Accepted |  |
| GB-152 |  | 7.24.1.6, 7.31.4.1.3 |  | te | The wording “may first round to M significant hexadecimal digits according to the applicable rounding direction mode” is unclear as to whether the applicable mode is the decimal mode (because this is a function for a decimal type), or the binary mode (because it’s a conversion between two hexadecimal strings). Using the decimal mode seems better; otherwise these functions would need to be affected by both FENV_ROUND and FENV_DEC_ROUND, including the possibility of one mode being dynamic and one being constant, if the implementation chooses to have a finite M. | Change “rounding direction mode” to “decimal rounding direction mode”, in both places. |  |  |
| GB-153 |  | 7.24.1.6, 7.31.4.1.3 |  | ed | “else s is set to 1” (paragraph 4 of both subclauses) should have “s” in italics. | Change “s” to italics (LaTeX $s$) in both places. | Accepted |  |
| GB-154 |  | 7.24.7 | 2 | ed | The wording “It is implementation-defined whether these functions avoid data races with other calls to the same function.” should have been appended to paragraph 1, not paragraph 2. | Move that wording from the end of paragraph 2 to the end of paragraph 1. | Accepted |  |
| GB-155 |  | 7.29.1 | 5 | ed | “The semantics of the other member and its normal range is expressed in the comments.”, about struct tm, should be restored to the wording it had in CD1; there have been incorrect changes to the wording apparently resulting from the tv_nsec sentence being wrongly mixed into this paragraph. | Change “The semantics of the other member and its normal range is expressed in the comments.” to “The semantics of the members and their normal ranges are expressed in the comments.”. | Accepted |  |
| GB-156 |  | 7.29.1 | 5 | ed | The description of tv_nsec should be immediately after the display of members of struct timespec, not mixed into the description of struct tm. | Move the sentence about the type of tv_nsec before the sentence “The tm structure shall contain at least the following members, in any order.”. The latter sentence should then start a new paragraph, with its own number. | Accepted |  |
| GB-157 |  | 7.29.2.1 | 3 | ed | “(clock_t)(-1)” should all be in a fixed-width font (as was fixed in many other places in CD2, but not here). | Change it to a fixed-width font. | Accepted |  |
| GB-158 |  | 7.29.2.3 | 3 | ed | “(time_t)(-1)” should all be in a fixed-width font (as was fixed in many other places in CD2, but not here). | Change it to a fixed-width font. | Accepted |  |
| GB-159 |  | 7.29.2.3 | 3 | te | AG Reference Bug 1614 (https://austingroupbugs.net/view.php?id=1614) ======== Further discussion in the Austin Group identified two additional problems with the mktime function. 1. When the tm components are range-corrected this can result in a tm_year value that is not representable as an int. The standard is silent about what happens in this case and therefore this is undefined behavior. All implementations we have tested either handle this in the same way as the return value not being representable, i.e. return (time_t)(-1), or don't detect the problem and return the calculated calendar time but with a bogus tm_year value. It would be good if returning (time_t)(-1) could be required, instead of the onus being on applications to avoid undefined behavior by ensuring tm_year will not overflow before they call mktime. 2. It is possible for the calendar time to be returned to have the value (time_t)(-1) and the standard does not provide any way for applications to distinguish a successful return of this value from the return-value-is-not-representable case. There is some existing practice where application code sets tm_wday to an out-of-range sentinel value and checks whether it was changed by mktime, and we are not aware of any implementation where this does not work. It would be good for this method to be required to work. | Under "Returns" in 7.29.2.3 para 3, change: If the calendar time cannot be represented in the time_t encoding used for the return value, the function returns the value (time_t)(-1). to: If the calendar time cannot be represented in the time_t encoding used for the return value or the value to be returned in the tm_year component of the structure pointed to by timeptr cannot be represented as an int, the function returns the value (time_t)(-1) and does not change the value of the tm_wday component of the structure. and in the example code change: if (mktime(&time_str) == (time_t)(-1)) time_str.tm_wday = 7; to: time_str.tm_wday = 7; mktime(&time_str); |  |  |
| GB-160 |  | 7.29.2.5 | 3 | ed | “(time_t)(-1)” should all be in a fixed-width font (as was fixed in many other places in CD2, but not here). | Change it to a fixed-width font. | Accepted |  |
| GB-161 |  | 7.29.3 | 2 | ed | “the following functions” should be “they” (to make it clear it’s just the two functions described in this paragraph, not any of the other following functions). This was part of the wording from N3072 item 9, but not marked there as an edit to the previous wording. | Change “the following functions” to “they”. | Accepted |  |
| GB-162 |  | 7.29.3.5 | 3 | ed | In the %F description, the duplicate “%F” at the start of the description should be removed, and “%Pm” should be “%m” (there’s no such strftime format as “%Pm”). (“%Pm” was introduced in commit 43b6f1d540cd87cb6019e406c5d448ce8916628f, probably accidentally.) | Remove the duplicate “%F” in “%F is equivalent” and change “%Pm” to “%m”. | Accepted |  |
| GB-163 |  | 7.31.3.3 | 3 | te | AG Reference Bug 1022 (https://austingroupbugs.net/view.php?id=1022) ========= The fgetwc() function was changed in C17 to require the error indicator for the stream to be set in the case of an encoding error. However, the equivalent requirement for fputwc() is not present, though POSIX requires it and does not mark it as an extension to C99. Since POSIX defers to C, and we are not aware of any implementations that do not set the error indicator, this should be aligned. | Change: "If an encoding error occurs, the value of the macro EILSEQ is stored in errno and fputwc returns WEOF." to: "If an encoding error occurs, the error indicator for the stream is set and the value of the macro EILSEQ is stored in errno and fputwc returns WEOF." |  |  |
| GB-164 |  | 7.31.4.1.2 | 5, 11 | ed | Paragraph 5 has two paragraphs, each of which should have its own number, and paragraph 11 has three paragraphs, each of which should have its own number. (This is part of CD1 comment GB-231, accepted but not fully fixed.) | Give these paragraphs their own numbers. | Accepted |  |
| GB-165 |  | 7.31.4.1.3 | 1 | ed | The third parameter of the functions should have type wchar_t ** restrict, not char ** restrict. | For all three functions, change “char” to “wchar_t” in the third parameter. | Accepted |  |
| GB-166 |  | 7.31.4.1.3 | 3 | ed | The bullet point for hexadecimal numbers should say “wide character” not “character” for the decimal point. | Change “decimal-point character” to “decimal-point wide character”. | Accepted |  |
| GB-167 |  | A.2.3 |  | ed | Stray strings “[-6ex]” appear in four places. (This is CD1 comment GB-253, accepted but not fixed.) | Remove those strings. | Accepted |  |
| GB-168 |  | A.4.1 |  | ed | “n-char-sequence:” should be shifted left. (The related cases of n-wchar-sequence and d-wchar-sequence were fixed following CD1 comments US-250 and US-251; the formatting of n-char-sequence in this Annex wasn’t mentioned in a CD1 comment, but is now clearly the odd one out.) | Shift “n-char-sequence:” left. | Accepted |  |
| US-169 |  | A.4.1 |  | ed | Page 470: "n-char-sequence:" is too far to right | Shift it left (like the others). | Accepted with comment: Resolved by GB-168. |  |
| GB-170 |  | Annexes C, E, I |  | ed | In Annexes not divided into clauses, the page footer shows a clause number from the previous Annex (for example, B.31 on page 498 for Annex C). This was CD1 comment GB-259, accepted but not fixed. | Fix the footers. | Accepted |  |
| GB-171 |  | B.12 |  | ed | The entry for __STDC_VERSION_SETJMP_H__ is badly formatted (overlaps other parts of the summary). | Fix the formatting. | Accepted |  |
| GB-172 |  | B.16 |  | ed | The entry for __STDC_VERSION_STDATOMIC_H__ is badly formatted (overlaps other parts of the summary). | Fix the formatting. | Accepted |  |
| GB-173 |  | B.19 |  | ed | The summary for stdckdint.h is missing all content other than __STDC_VERSION_STDCKDINT_H__; instead they are misplaced under stdnoreturn.h (B.24) | Move the rest of the header content from B.24. | Accepted |  |
| FR-174 |  | B.19 |  | ed | no type-generic function is listed | list the type-generic functions | Accepted |  |
| GB-175 |  | B.23 |  | ed | The stdlib.h summary is missing strtod32, strtod64, strtod128. | Add those to the summary (alongside the corresponding strfromdN functions, i.e. conditional on DFP support). | Accepted |  |
| GB-176 |  | B.23 |  | ed | The stdlib.h summary is missing once_flag, ONCE_FLAG_INIT and call_once. This was CD1 comment GB-262 (though that omitted to note the absence of ONCE_FLAG_INIT from the summary), accepted but not fixed. | Add those to the summary. | Accepted |  |
| GB-177 |  | B.23 |  | ed | The entry for __STDC_VERSION_STDLIB_H__ is badly formatted (overlaps other parts of the summary). | Fix the formatting. | Accepted |  |
| FR-178 |  | B.23 |  | ed | The strtodN functions are missing. | Add their synopsis to the conditional section concerning DFP. | Accepted with comment: Resolved by GB-175. |  |
| GB-179 |  | B.26 |  | ed | The entry for __STDC_VERSION_TGMATH_H__ is badly formatted (overlaps other parts of the summary). | Fix the formatting. | Accepted |  |
| GB-180 |  | B.29 |  | ed | The entry for __STDC_VERSION_UCHAR_H__ is badly formatted (overlaps other parts of the summary). | Fix the formatting. | Accepted |  |
| GB-181 |  | B.30 |  | ed | The wchar.h summary is missing wcstod32, wcstod64, wcstod128. | Add those to the summary (conditional on DFP support). | Accepted with comment: Use the proposed change plus wording from N 3135 with wcstol -> wcstold. |  |
| GB-182 |  | B.5 |  | ed | This summary does not mention __STDC_VERSION_FENV_H__. | Add __STDC_VERSION_FENV_H__ to this summary. | Accepted |  |
| GB-183 |  | B.7 |  | ed | This summary does not mention the new macros for %b or the optional %B. | Add those macros to the summary. | Accepted |  |
| FR-184 |  | B.7 |  | ed | Some macros are missing in the listing. | Add the mandatory macros for the b format specifiers. Create a new paragraph for optional specifiers and list the new B format specifiers, there. | Accepted |  |
| GB-185 |  | B.9 |  | ed | The entry for __STDC_VERSION_LIMITS_H__ is badly formatted (overlaps other parts of the summary). | Fix the formatting. | Accepted |  |
| US-186 |  | F.10.13 | Para 1 | ed | It might not be clear that NaN payloads do not need to be considered in determining whether the values of one type include the values of another type | To: “The payload is intended for implementation-defined diagnostic information about the NaN, such as where or how the NaN was created.” attach the footnote: “*) For purposes of determining value inclusion (as in 6.2.5, 7.12 and H.11), quiet NaN representations can be regarded as having the same value, regardless of payloads.” | Accepted |  |
| US-187 |  | F.10.13 | Para 2 | ed | Wording has not been updated to accommodate decimal floating types. | Change: “… details of IEC 60559 formats that might not be supported if __STDC_IEC_60559_BFP__ is not defined.” to: “… details of IEC 60559 formats that might not be supported if the relevant feature test macro, __STDC_IEC_60559_BFP__ or __STDC_IEC_60559_DFP__, is not defined.” | Accepted |  |
| GB-188 |  | F.10.3.7 | 4 | ed | “isinfinite” should be “isfinite” (the fix for CD1 comment GB-268 was applied incorrectly). | Change “ininfinite” to “isfinite”. | Accepted with comment: Change “isinfinite” to “isfinite”. |  |
| US-189 |  | F.10.3.7 | Para 1, 4 | ed | F.10.3.7 uses “exp” while 7.12.6.7 uses “p” for the same argument for the same function, frexp. This is confusing because the code in F.10.3.7 para 4 is said to be a possible body for the frexp function whose prototype is given in 7.12.6.7. | Change all (8) instances of “exp” in F.10.3.7 to “p”. | Accepted |  |
| GB-190 |  | F.10.4.2 | 1 | ed | In the last two compoundn bullet points, “±” should be “+” (the fix for CD1 comment GB-269 was applied incorrectly). | Change “±” to “+” (in both those bullet points). | Accepted |  |
| US-191 |  | F.10.4.2 | Para 1 | ed | In the last two bullets, the first argument should be “+∞” not “±∞”. Ref. CD NB comment GB-269. | In the last two bullets, change the first argument from “±∞” to “+∞”. | Accepted with comment: Resolved by GB-190. |  |
| GB-192 |  | F.10.4.6 | 1 | ed | “signalling” should be “signaling” for consistency with spelling used elsewhere in the document. | Change “signalling” to “signaling”. | Accepted |  |
| GB-193 |  | F.10.6.8 | 1 | ed | The changes made here to address CD1 comments US-272 / GB-273 leave the text stylistically inconsistent with all the other function specifications here. | Change the bullet points back to the form they had in CD1, removing the “roundeven returns” text above the two bullet points. Address those CD1 comments by changing \pnum to \inum in the LaTeX source instead, as for other such subclauses that start with a bulleted list. | Accepted |  |
| GB-194 |  | F.10.8.7 | 1 | ed | In “The canonicalize functions”, “canonicalize” should be in a fixed-width font. | Change “canonicalize” into a fixed-width font. | Accepted |  |
| US-195 |  | F.2 | Para 1, Fn 441 | te | Since Annex F allows long double to have a non-IEC format, the question might arise, is such a long double type required to have signed zeros and signed infinites? The following change clarifies that the answer is, yes. This comment does not suggest a similar change for implementations that do not conform to Annex F because the behavior of signed and unsigned zeros and infinities is so loosely specified for such implementations that the way those values are regarded for determining whether the values of one type are included in the values of another type can be left to the implementation. | In F.2 #1, footnote 441 (ref N3095), change: “A non-IEC 60559 long double type is required to provide infinity and NaNs, as its values include all double values.” to: “A non-IEC 60559 long double type is required to provide signed infinities, signed zeros, and NaNs, as its values must include all double values.” |  |  |
| GB-196 |  | F.8.4 | 3 | ed | “initialization” should be “initializations”, as shown in the CD1 GB-279 changes in N3082. | Change “initialization” to “initializations”. | Accepted |  |
| GB-197 |  | F.8.5 | 1 | te | As in the accepted changes for CD1 GB-279 in N3082, constexpr initializations should be included here with static and thread storage duration. | After “static or thread storage duration” insert “or that are declared with storage-class specifier constexpr”. | Accepted |  |
| GB-198 |  | F.8.5 | 3 | ed | “conversions is” should be “conversion is”. | Change “conversions is” to “conversion is”. | Accepted |  |
| US-199 |  | F.8.5 | Para 1 | ed | The change in this section proposed in N3082 to address CD NB comment GB-151 was not made. (All the other changes for this comment were made.) | In F.8.5 #1 insert after “of objects that have static or thread storage duration” the words “or that are declared with storage-class specifier constexpr”. | Accepted with comment: Resolved by GB-197. |  |
| US -200 |  | F.9.1 | 3 | ed | Page 516: Wrong place for action. | "between function calls" -> "between sequence points" | Rejected with comment: Clarify as shown in N 3135 instead of the proposed change. |  |
| US-201 |  | G.4.3 | 2 | ed | Page 536: Missing mention of signaling NaN. | "discarded" -> "discarded (even if it is a signaling NaN)" | Rejected |  |
| GB-202 |  | G.5.2 | 2 | ed | In the top left cell of this table, “+” and “-” represent C operators (as illustrated by the previous tables for “*” and “/”) so should be in a fixed-width font | Change that “+” and “-” (only the ones in the top left cell, not any of the others elsewhere in the table) into a fixed-width font. | Accepted |  |
| US-203 |  | H.11 | Para 6 | te | The definitions of the long_double_t, _FloatN_t, and _DecimalN_t types refer to one type as having at least the range and precision of another type. This does not imply inclusion for all the values (e.g. NaNs and infinities) in the second type. The proposed specification (similar to that in 6.2.5 for the standard floating types) addresses this problem. | Change: “These are floating types, such that: — each of the types has at least the range and precision of the corresponding real floating type; — long_double_t has at least the range and precision of double_t; — _FloatN_t has at least the range and precision of _FloatM_t if N > M; — _DecimalN_t has at least the range and precision of _DecimalM_t if N > M.” to: “These are floating types, such that: — the values of long double are a subset of the values long_double_t; — the values of _FloatN are a subset of the values _FloatN_t; — the values of _DecimalN are a subset of the values _DecimalN_t; — the values of double_t are a subset of the values of long_double_t; — the values of _FloatM_t are a subset of the values of _FloatN_t if M < N; — the values of _DecimalM_t are a subset of the values of _DecimalN_t if M < N.” |  |  |
| US-204 |  | H.12.2 | Para 4 | ed | The EXAMPLE has a locale dependency that is not mentioned. In particular, determination of the value of MAXSIZE assumes the decimal-point is just one byte. | Change: #define MAXSIZE 41 // > intermediate hex string length to: #define MAXSIZE 41 // > intermediate hex string length for “C” locale and change: “The array s for the output of strfromencf128 need have no greater size than 41, ...” to: “Assuming a single-byte decimal-point character as in the “C” locale, the array s for the output of strfromencf128 need have no greater size than 41, ...” | Accepted |  |
| GB-205 |  | H.12.3.1, H.12.3.2, H.12.4.1, H.12.4.3, H.12.4.4 |  | ed | “in the arrays” should be “in the array” , in each of these subclauses, because only a single array with byte order considerations is used by these functions (see CD1 comment GB-173). There is also a missing space after “arrays” in H.12.4.1. | In each of these five places, change “in the arrays” to “in the array”, also adding the missing space in H.12.4.1. | Accepted |  |
| US-206 |  | H.12.3.1, H.12.4.1 | 2 | ed | In these sections, “order of bytes in the arrays” should be changed to “order of bytes in the array”, as prescribed in CD NB comment GB-173. | Change “order of bytes in the arrays” to “order of bytes in the array”. | Accepted with comment: Resolved by GB-205. |  |
| GB-207 |  | H.12.4 | 1 | ed | “both” should be removed now there are four function families listed here not two. | Remove “both”. | Accepted |  |
| US-208 |  | H.12.4.1 | 2 | ed | A space is missing in “arraysfollows”. | Change “arraysfollows” to “arrays follows”. | Accepted |  |
| US-209 |  | H.12.4.4 | 1 | ed | The parameters nptr and endptr in both functions in the Synopsis do not have wide character types as prescribed in N3105. I.e., they use char instead of wchar_t. Do not change the first parameter encptr which correctly uses char. | Change: void wcstoencdecdN(unsigned char encptr[restrict static N/8], const char * restrict nptr, char ** restrict endptr); void wcstoencbindN(unsigned char encptr[restrict static N/8], const char * restrict nptr, char ** restrict endptr); to: void wcstoencdecdN(unsigned char encptr[restrict static N/8], const wchar_t * restrict nptr, wchar_t ** restrict endptr); void wcstoencbindN(unsigned char encptr[restrict static N/8], const wchar_t * restrict nptr, wchar_t ** restrict endptr); | Accepted |  |
| GB-210 |  | H.13 | 6 | ed | “undefined type is undefined” should just be “undefined”; the extra words don’t seem meaningful or helpful. | Change “undefined type is undefined” to “undefined”. | Accepted |  |
| GB-211 |  | H.3 | 7 | ed | In the TRUE_MIN description, “min” should not be in italics. (This is the remaining unfixed part of CD1 comment GB-300; the other parts were fixed.) | Change “min” out of italics (LaTeX \min). | Accepted |  |
| US-212 |  | Index |  | ed | Page 725: Extra entry | Remove “MAX identifier suffix, 140” | Accepted |  |
| US-213 |  | Index |  | ed | Page 715: Extra entry | Remove “MAX, 140” | Accepted |  |
| US-214 |  | Index |  | ed | Page 715: Missing entries | Add to identifier suffix: _DECIMAL_DIG _DIG _EPSILON _H__ _MANT_DIG _MAX_10_EXP _MAX_EXP _MIN_10_EXP _MIN_EXP _SNAN _TRUE_MIN _r | Accepted |  |
| US-215 |  | Index |  | ed | Page 714: Wrong classification of entries | Move from identifier prefix to identifier suffix _DECIMAL_DIG _H__ _r | Accepted |  |
| US-216 |  | Index |  | ed | Page 697: Duplicate entry | Merge the two “IEC 60559” entries | Accepted |  |
| US-217 |  | Index |  | ed | Page 690: Extra entry | Remove all __suffix__ entries | Accepted |  |
| US-218 |  | Index |  | ed | Page 690: Extra entry | Remove all __prefix__ entries | Accepted |  |
| US-219 |  | Index |  | ed | Page 679: _r entry wrong | _r identifier prefix -> _r identifier suffix | Accepted |  |
| US-220 |  | Index |  | ed | Page 679: Extra entry | Remove all __suffix__ entries | Accepted with comment: Also remove the index entry for page 610. |  |
| US-221 |  | Index |  | ed | Page 679: Extra entry | Remove all __prefix__ entries | Accepted with comment: Also remove the index entry for page 610. |  |
| US-222 |  | Index |  | ed | Page 678: _H__ entry wrong | _H__ identifier prefix -> _H__ identifier suffix | Accepted |  |
| US-223 |  | Index |  | ed | Page 678: _DECIMAL_DIG entry wrong | _DECIMAL_DIG identifier prefix -> _DECIMAL_DIG identifier suffix | Accepted |  |
| US-224 |  | Index |  | ed | Page 610: Extra entry (only mentioned in J.6.1) | Remove all references to __suffix__ | Accepted |  |
| US-225 |  | Index |  | ed | Page 610: Extra entry (only mentioned in J.6.1) | Remove all references to __prefix__ | Accepted |  |
| US-226 |  | Index |  | ed | Page 609: Extra entry | “_Float” should be removed. | Accepted |  |
| US-227 |  | Index |  | ed | Page 609: Extra entry | "_EXT__" should be removed. | Accepted |  |
| US-228 |  | Index |  | ed | Page 609: Extra entry | “_Decimal” should be removed. | Accepted |  |
| GB-229 |  | J.2 |  | ed | Commit 23cd2a52f91117370ddbf4885294f945278403a4 removed an undefined behavior entry, “The initializer for a structure or union object that has automatic storage duration is neither an initializer list nor a single expression that has compatible structure or union type (6.7.10).”. But the requirement concerned here is still in Semantics, not Syntax or Constraints, so violating it is still undefined behavior and should be listed as such. All that was requested in CD1 comment GB-083 (accepted) was to remove “that has automatic storage duration” from the wording of that item, not to remove that item completely. | Between the present items 80 and 81, restore the previous item 81, minus the words that should have been removed: “The initializer for a structure or union object is neither an initializer list nor a single expression that has compatible structure or union type (6.7.10).”. | Accepted |  |
| GB-230 |  | J.2 |  | ed | Item 80 needs updating for empty initializers. | In item 80, change “neither a single expression” to “neither a single expression, an empty initializer,”. | Accepted |  |
| GB-231 |  | J.2 |  | ed | Item 54 does not reflect changes to the definition of constant expressions. This was CD1 comment GB-314, accepted but not applied. | Change “enumeration constants” to “named and compound literal constants of arithmetic type”. | Accepted |  |
| US-232 |  | J.2 | 1 | ed | Freeing a null pointer is unconditionally allowed in 7.24.3.3 The free function, but this is not correctly depicted in the UB description in J.2 | Replace: (181) The pointer argument to the free or realloc function does not match a pointer earlier returned by a memory management function, or the space has been deallocated by a call to free or realloc (7.24.3.3, 7.24.3.7). with: (181) The pointer argument to the free or realloc function is unequal to a null pointer and does not match a pointer earlier returned by a memory management function, or the space has been deallocated by a call to free or realloc (7.24.3.3, 7.24.3.7). | Accepted |  |
| GB-233 |  | J.3.12 | 1 | ed | Item 14 (“The byte order of decimal floating type encodings (7.12.16).”) should be removed, as per CD1 comment GB-173, now that the functions referenced are defined to use the __STDC_ENDIAN_NATIVE__ ordering. | Remove item 14. | Accepted |  |
| GB-234 |  | K.3.9.2.2.1 | 3 | ed | CD1 comment US-328 reported “Bad line break and inconsistent use of space in front of operator parentheses.”, and was accepted but no change seems to have been applied. | The requested / accepted change was: Remove space after “sizeof”, which should fix the bad line break there. | Accepted |  |
| GB-235 |  | M.1 | 2 | ed | “may use this table know” is ungrammatical. | Change “may use this table know” to “may use this table to know”. | Accepted |  |
| GB-236 |  | M.1 | Table Annex M.1 | ed | The noreturn attribute was introduced with value 202202L, not 202002L. | Change “202002L” to “202202L” in the first noreturn row. | Accepted |  |
| GB-237 |  | M.1 | Table Annex M.1 | ed | The word “Annex” should not appear in the numbering of a table; it should be Table M.1 not Table Annex M.1. (See the ISO/IEC Directives, Part 2, 20.4 Numbering and subdivision.) | Change “Table Annex M.1” to “Table M.1”. | Accepted |  |
| FR-238 |  | overall |  | te | ISO has recently rejected TS 6010 claiming “content not appropriate for an ISO deliverable”. Since the votes on this subject had been conditioned under the premise that such a TS would be possible, the vote should be redone. Also at least two years have passed now since we have publicly announced to the C community that we would like to go in that direction. The texts concerning this are available in WG14 documents even longer. The premature ballot procedure that we already had in March only found one little glitch in the proposed text itself. | Integrate the text of N3057 Annex C into C23, amended by one additional change corresponding to the last line of N3117. | *Defer to get input from Martin see also DE-002 |  |
| GB-239 |  | throughout |  | ed | The formatting of footnote references is inconsistent throughout the draft; sometimes the footnote number appears before punctuation, sometimes after, sometimes preceded by a space, sometimes not. This was CD1 comment GB-338, which was accepted; some instances have been fixed, but certainly not all. Footnote 5 is the first example in CD2 of a footnote where the footnote number is preceded by a space. Footnote 10 is the first example where punctuation follows the footnote number. | Change to follow usual practice consistently: no space before the footnote number, and, if adjacent to punctuation, coming after rather than before that punctuation. | Accepted |  |

ISO_IEC CD 9899.2_AFNOR.doc: Collation successful

ISO_IEC CD 9899.2_ANSI.docx: Collation successful

ISO_IEC CD 9899.2_BSI.doc: Collation successful

ISO_IEC CD 9899.2_DIN.docx: Collation successful

ISO_IEC CD 9899.2_SCC.docx: Collation successful

Collation of files was successful. Number of collated files: 5

SELECTED           (number of files):  5

PASSED TEST        (number of files conformed to CCT table model):  5

FAILED TEST        (number of files conformed to CCT table model):  0

CCT - Version 2021.1